NOLO SDK For Windows

Documentation

SDK Version：V 2.2

NOLO Co., Ltd

November 2018

目录

一、 简介	3

1.1 关于NOLO	3

1.2 关于NOLO CV1	3

1.3 关于NOLO SDK For Windows	3

1.4 关于NOLO HOME PC版	4

二．接入准备	5

2.1 NoloServer目录	5

2.2 NoloClient 目录	6

三．接口说明	7

3.1基本函数	7

3.2定位信息获取	8

四．注意事项	10

4.1 开发环境	10

4.2 管理员权限	10

- Overview

1.1 About NOLO

NOLO is dedicated to combine desktop-grade VR gaming experience with the convenience of mobile VR devices, redefining a mobile VR gaming experience like never before.

NOLO kit is compatible with some 87,000,000 VR headsets of all kinds currently on the market, indicating huge market potential. In addition, we’ve partnered with VR headset companies, robotic companies, and drone companies around the globe.

1.2 About NOLO CV1

NOLO CV1 is a short-range high-accuracy motion tracking equipment kit for VR/AR gaming systems, composed of a Base Station, a headset marker and 2 controllers.

NOLO CV1 is compatible with all mainstream mobile VR headsets, some PC VR/AR headsets, and All-In-One’s based on NibiruOS, providing position information and interaction functionality to these devices. With simple setup at first launch, users can move around in the virtual world like in real world, and interact with virtual objects using our hand controllers.

最新的NOLO CV1固件版本，相对于老版本的固件，已经做了较大的优化，包括延迟，稳定性，易用性，都有了明显的提高。

1.3 About NOLO SDK For Windows

NOLO SDK For Windows, developed by NOLO Inc., is the SDK for developers in Windows to acquire NOLO device data, and mainly applied to connect with each headset manufacturer to work with NOLO device as well as access to SteamVR. Moreover, developers may integrate NOLO HOME into their own PC softwares for the purpose of monitoring the connection and the amount of electricity of NOLO device.

1.4 About NOLO HOME For PC

NOLO HOME PC client is the VR eco-platform of NOLO brand, and the Windows interface client developed by NOLO Inc. Currently, its core functions have integrated all the features of NOLO SDK For Windows. For part of manufacturers and individuals without their own client, NOLO HOME PC Edition can directly be used as NOLO device management software, and take advantage of all the functions. In addition to the basic Server function, current version of NOLO HOME PC integrates the compatibility of MI VR standalone headsets and Huawei VR2 standalone headsets.

Additionally, a great advantage of using NOLO HOME PC client is that one can enjoy the new product experience provided by NOLO in the most convenient way without delay.

Manufacturers and individual developers who do not need interface clientmay also usethis SDK to customize and develop their own NOLO device management tools and SteamVR driver.

Developers shall notice that Server side is single instance schema, and take the latest version of NoloServer.exe as the exclusive service provider. For example, when a client invokes a NoloServer.exe V1.0.1, and another client invokes NoloServer.exe V1.0.3, a higher version, NoloServer.exe V1.0.1 will automatically end. All the clients will acquire data from NoloServer.exe V1.0.3.

2．Set Up Development Environment

NOLO SDK adopts C/S architecture design, takes Noloserver.exe as Server, and is responsible for reading NOLO device data and processing. NoloClientLib.dll is Client, and SDK acquires NOLO CV1 device data by establishing communication between libzmq and Noloserver.exe. Game itself does not need to read USB device data, and only has to establish connection with accessed NOLO server to realize data receiving and sending.

The directory structure of NOLO SDK For Windows officially acquired from NOLO is as follows:

2.1 NoloServer Directory

NoloServer is server program. When a client invokes StartNoloServer, NoloServer.exe will be started. By default, it is not necessary to specify a path when NoloServer folder is put into the peer directory of NoloClientLib.dll. Otherwise, it is necessary to specify all paths of NoloServer.exe.

NoloServer

|---------jsoncpp.dll

|---------libzmq-64.dll

|---------Nolo_Device.dll

|---------Nolo_Device.lib

|---------NoloServer.exe

2.2 NoloClient Directory

32-bit library is under lib32 subdirectory and 64-bit library is under lib64 directory. NoloClientAPI directory contains header file of Client.

NoloClient

|-------lib32

|      |--------libzmq-32.dll

|      |--------NoloClientLib.dll

|      |--------NoloClientLib.lib

|

|-------lib64

|      |--------libzmq-64.dll

|      |--------NoloClientLib.dll

|      |--------NoloClientLib.lib

|

|------NoloClientAPI

|        |-------Nolo_Math.h

|        |-------Nolo_DeviceType.h

|        |-------NoloClientLib.h

3．API Description

3.1 Basic function

| function name | bool StartNoloServer(const wchar_t *StrServerPath = L""); |
| --- | --- |
| function description | Initiate NoloServer.exe under specified directory. |
| input parameters | const wchar_t *StrServerPath = L"" |
| return value | bool |
| prerequisites | NoloServer exists in specified directory. |

| function name | void SetEventListener(INOLOZQMEvent *Listener); |
| --- | --- |
| function description | Set NOLO device event monitoring pointer, and for the details of event please see INOLOZQMEvent. |
| input parameters | INOLOZQMEvent *Listener |
| return value | void |
| prerequisites | NONE |

| function name | void SetHmdCenter(const NVector3 &hmdCenter); |
| --- | --- |
| function description | Setting of headset rotation center transformation |
| input parameters | hmdCenter【Input】displacement vector value from [midpoint between eyes] to [headset marker] |
| return value | void |
| prerequisites | It is required to set the value to convert head position when head position shall be used. Vector coordinate system: x positive direction: Right, y positive direction: Up, z positive direction: forward Example value: (0.00f,0.08f,0.08f) |

| function name | bool OpenNoloZeroMQ(); |
| --- | --- |
| function description | Open ZMQ data monitoring, and receive data sent from Server |
| input parameters | INOLOZQMEvent *Listener |
| return value | bool |
| prerequisites | NONE |

| function name | Void CloseNoloZeroMQ(); |
| --- | --- |
| function description | Close ZMQ data monitoring |
| input parameters | void |
| return value | void |
| prerequisites | NONE |

| function name | void TriggerHapticPulse(ENoloDeviceType deviceType, int intensity); |
| --- | --- |
| function description | Send vibration instruction to specified controller |
| input parameters | @ deviceType 【Input】 controller type, Valid value: eLeftController,eRightController @ intensity 【Input】 vibration strength, from weak to strong: 50 – 100 |
| return value | void |
| prerequisites | NoloServer.exe has been initiated |

3.2 Acquire position information

| function name | NOLOData GetNoloData() |
| --- | --- |
| function description | Acquire all the latest NOLO data |
| input parameters | void |
| return value | NOLOData |
| prerequisites | NoloServer.exe has been initiated |

| function name | Controller GetLeftControllerData(); |
| --- | --- |
| function description | Acquire the latest left controller data |
| input parameters | void |
| return value | Controller |
| prerequisites | NoloServer.exe has been initiated |

| function name | Controller GetRightControllerData(); |
| --- | --- |
| function description | Acquire the latest right controller data |
| input parameters | void |
| return value | Controller |
| prerequisites | NoloServer.exe has been initiated |

| function name | HMD GetHMDData(); |
| --- | --- |
| function description | Acquire the latest NOLO headset marker data |
| input parameters | void |
| return value | HMD |
| prerequisites | NoloServer.exe has been initiated |

4．Attentions

4.1 Development Environment

Compiling environment of library file is VS2017, and the version of Dll offered is 64-bit Release Edition.

Currently, an example of head driver has been provided. (Huawei headset is used as display device)

Huawei headset driver is not provided temporarily for confidentiality reasons, and sample code is provided for reference only.

4.2 Administrator Permission

To block pop-up windows of Windows firewall, NoloServer.exe has obtained administrator permission.